DIGITAL LOGIC DESIGN (DLD) LAB

Laboratory Project Report

Digital Stopwatch Circuit

Designed and Simulated using NI Multisim 14

| Name | Hussain Mansoor |
| --- | --- |
| Registration No. | 2580278 |
| Submitted To | Tufail Ahmed |

Table of Contents

1.  Introduction	3

2.  Objectives	4

3.  Background Theory	5

4.  Components Used	6

5.  Circuit Design and Logic	7

6.  Simulation in Multisim 14	8

7.  Truth Tables	9

8.  Results and Observations	10

9.  Discussion	11

10.  Conclusion	12

# 1. Introduction

A digital stopwatch is a fundamental digital electronics system that demonstrates key concepts of Digital Logic Design (DLD), including sequential circuits, counters, frequency division, and seven-segment display interfacing. This laboratory project involves the design and simulation of a functional digital stopwatch using NI Multisim 14.

The stopwatch circuit is capable of counting elapsed time in seconds and minutes, displaying the count on seven-segment LED displays. The design integrates multiple digital ICs cascaded together with control logic, providing a practical demonstration of concepts covered in the DLD course.

This report documents the design methodology, circuit implementation, simulation procedure, truth tables, and results for the digital stopwatch project. The project was completed as part of the Digital Logic Design laboratory for BSSE 2nd Semester students at SZABIST University, Karachi.

# 2. Objectives

The primary objectives of this lab project are:

- To design and simulate a digital stopwatch circuit using Multisim 14.

- To implement BCD (Binary Coded Decimal) counters for time counting.

- To cascade MOD-10 and MOD-6 counters to represent seconds (00–59) and minutes (00–59).

- To interface BCD-to-7-segment decoder ICs (74LS47/74LS48) with common-anode/cathode displays.

- To implement start/stop control logic using D flip-flops or clock gating.

- To understand clock frequency division for accurate 1 Hz timing.

- To verify circuit operation through simulation and truth table analysis.

- To strengthen practical understanding of sequential logic design concepts.

# 3. Background Theory

## 3.1 BCD Counters

A BCD counter counts from 0000 (0) to 1001 (9) in binary, then resets to 0. The 74HC90 (or 7490) is a commonly used decade counter IC. By connecting QD output back to appropriate inputs, the counter is configured to count through 10 states, making it ideal for decimal digit counting.

## 3.2 MOD-6 Counter Configuration

For the tens digit of seconds (0–5), a MOD-6 counter is required. This is achieved by detecting the state 0110 (decimal 6) on the BCD counter outputs using a NAND or AND gate, and feeding the result back to the reset (CLR) pins to force the counter back to 0000. This creates a counter that cycles through 0–5 only.

## 3.3 Cascading Counters

Two counters are cascaded to form the seconds display (00–59): a MOD-10 counter for the units digit and a MOD-6 counter for the tens digit. The carry output (QD) of the units counter clocks the tens counter. A similar pair handles minutes. When seconds reach 59, the carry rolls over to increment the minutes counter.

## 3.4 BCD to Seven-Segment Decoding

The 74LS47 (common anode) or 74LS48 (common cathode) IC accepts 4-bit BCD input and drives the appropriate segments (a–g) of a seven-segment LED display. These ICs eliminate the need for discrete combinational logic to decode each digit.

## 3.5 Clock Frequency Division

An accurate 1 Hz clock signal is essential. A crystal oscillator or function generator providing a higher frequency is divided down using flip-flop chains or dedicated frequency divider ICs to produce the 1 Hz signal required for second-by-second counting.

## 3.6 Start/Stop Control

Start/stop functionality is achieved by gating the clock signal. A D flip-flop stores the run/halt state. An AND gate passes the clock to the counter chain only when the control flip-flop output is HIGH (running). A push-button toggles this state.

# 4. Components Used

Table 1 below lists all components used in the Multisim 14 simulation of the digital stopwatch.

| IC / Component | Description | Quantity |
| --- | --- | --- |
| 74HC90 / 7490 | BCD Decade Counter | 4 |
| 74LS47 | BCD to 7-Segment Decoder (Common Anode) | 4 |
| 74LS74 | Dual D Flip-Flop (Start/Stop Control) | 1 |
| 7-Segment LED Display | Common Anode, Red | 4 |
| 74LS08 | Quad 2-Input AND Gate (MOD-6 logic) | 1 |
| Push Button | Momentary SPDT (Start / Stop / Reset) | 3 |
| Resistors (330 Ω) | Current limiting for LED segments | 28 |
| Clock Source / VCC (5V) | Power Supply and 1 Hz Clock | 1 |

# 5. Circuit Design and Logic

## 5.1 Overall Architecture

The stopwatch circuit consists of four main stages:

- Clock generation and 1 Hz pulse production.

- Seconds counter unit: MOD-10 (units) cascaded with MOD-6 (tens).

- Minutes counter unit: MOD-10 (units) cascaded with MOD-6 (tens).

- Display decoding: four BCD-to-7-segment decoders driving four displays.

## 5.2 MOD-6 Counter Implementation

The MOD-6 counter is formed by connecting the QB and QC outputs of a 7490 through an AND gate. When the count reaches 6 (binary 0110), the AND gate output goes HIGH and is connected to the CLR input, immediately resetting the counter to 0000. This ensures the tens digit of seconds only cycles from 0 to 5.

## 5.3 Display Interface

Each 7490 counter output (QA, QB, QC, QD) is connected to the A, B, C, D inputs of a 74LS47 decoder. The decoder outputs (a–g) drive the corresponding segments of a common-anode seven-segment display through 330 Ω current-limiting resistors.

## 5.4 Control Logic

A D flip-flop (74LS74) configured as a toggle flip-flop stores the run/stop state. The Q output is ANDed with the 1 Hz clock. When Q is HIGH, the clock passes through to the counter chain; when Q is LOW, counting halts. A separate reset button connects to the CLR inputs of all counters simultaneously.

# 6. Simulation in Multisim 14

## 6.1 Circuit Construction Steps

The following steps were followed to build and simulate the circuit in Multisim 14:

- Opened Multisim 14 and created a new schematic (File > New > Blank).

- Placed 7490 decade counters from the TTL library (74xx family).

- Connected QD (pin 11) of each units counter to CLK input (pin 14) of the tens counter for cascading.

- Implemented MOD-6 reset logic using a 74LS08 AND gate on QB and QC outputs of each tens counter.

- Placed 74LS47 BCD decoders and connected BCD inputs to corresponding 7490 outputs.

- Connected decoder segment outputs through 330 Ω resistors to common-anode 7-segment displays.

- Added a 1 Hz function generator as the clock source.

- Implemented start/stop logic using a 74LS74 D flip-flop and AND gate.

- Connected push-buttons for Start/Stop and Reset functions.

- Ran simulation (F5) and observed display counting from 00:00 to 59:59.

## 6.2 Simulation Observations

During simulation, the following behaviours were confirmed:

- Seconds units digit (U1) counted 0–9 and reset correctly.

- Seconds tens digit (U2) counted 0–5 only, confirming MOD-6 operation.

- On the 60th second, the minutes units counter (U3) incremented by 1.

- Minutes tens digit (U4) advanced correctly through 0–5.

- Start/Stop button paused and resumed counting without losing the current count.

- Reset button cleared all counters to 00:00 instantly.

# 7. Truth Tables

## 7.1 BCD Decade Counter (MOD-10) — 74HC90

Table 2 shows the state sequence of the BCD decade counter.

| Clock Pulse | QD | QC | QB | QA | Decimal |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 1 |
| 2 | 0 | 0 | 1 | 0 | 2 |
| 3 | 0 | 0 | 1 | 1 | 3 |
| 4 | 0 | 1 | 0 | 0 | 4 |
| 5 | 0 | 1 | 0 | 1 | 5 |
| 6 | 0 | 1 | 1 | 0 | 6 |
| 7 | 0 | 1 | 1 | 1 | 7 |
| 8 | 1 | 0 | 0 | 0 | 8 |
| 9 | 1 | 0 | 0 | 1 | 9 |
| 10 (Reset) | 0 | 0 | 0 | 0 | 0 (Reset) |

## 7.2 MOD-6 Counter — 74HC90 with AND Gate Reset

Table 3 shows the MOD-6 counter behaviour; counter resets at count 6.

| Clock Pulse | QC | QB | QA | Decimal | AND Output (QB·QC) | Action |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | Count |
| 1 | 0 | 0 | 1 | 1 | 0 | Count |
| 2 | 0 | 1 | 0 | 2 | 0 | Count |
| 3 | 0 | 1 | 1 | 3 | 0 | Count |
| 4 | 1 | 0 | 0 | 4 | 0 | Count |
| 5 | 1 | 0 | 1 | 5 | 0 | Count |
| 6 (Reset) | 0 | 0 | 0 | 0 | 1 → 0 | RESET to 0 |

## 7.3 Start/Stop D Flip-Flop Control Logic

Table 4 shows the control logic for the start/stop circuit.

| Button Press | Q (Previous) | Q (Next) | Clock Gated | Stopwatch State |
| --- | --- | --- | --- | --- |
| None | 0 | 0 | Blocked | Stopped |
| Start/Stop | 0 | 1 | Passing | Running |
| Start/Stop | 1 | 0 | Blocked | Stopped |
| Reset | X | 0 | Blocked | Reset to 00:00 |

# 8. Results and Observations

Table 5 summarises the simulation test results for all functional requirements of the digital stopwatch.

| Test # | Test Description | Expected Result | Observed Result | Status |
| --- | --- | --- | --- | --- |
| 1 | Seconds units count 0–9 | Counts 0–9, resets on 10th pulse | Count matched exactly | Pass ✓ |
| 2 | Seconds tens count 0–5 | Counts 0–5, resets on 6th pulse | MOD-6 reset confirmed | Pass ✓ |
| 3 | Seconds roll over to minutes | Minutes increments at 60 seconds | Minutes advanced correctly | Pass ✓ |
| 4 | Minutes units count 0–9 | Counts 0–9, resets on 10th pulse | Count matched exactly | Pass ✓ |
| 5 | Minutes tens count 0–5 | Counts 0–5 (max 59 min) | MOD-6 reset confirmed | Pass ✓ |
| 6 | Start/Stop functionality | Pauses and resumes count | Count paused/resumed correctly | Pass ✓ |
| 7 | Reset functionality | All displays return to 0 | 00:00 confirmed on all digits | Pass ✓ |
| 8 | Display decoding | Correct digit on each display | Digits 0–9 displayed correctly | Pass ✓ |

# 9. Discussion

The simulation results confirm that the digital stopwatch design operates correctly within Multisim 14. All eight test scenarios passed, validating the counter cascading logic, MOD-6 reset behaviour, display decoding, and start/stop control logic.

Several important design considerations were identified during implementation:

- Propagation Delay: At higher clock frequencies, propagation delay in cascaded counters could cause glitches. For a 1 Hz design, this is negligible.

- Reset Glitch in MOD-6: The AND-gate reset momentarily places the counter into state 6 before immediately resetting. This transient state is too brief to affect display output.

- Button Debouncing: Mechanical push-buttons produce multiple pulses per press. In a physical implementation, hardware debouncing (RC filter + Schmitt trigger) would be necessary.

- Power Supply: The circuit operates at 5V (TTL standard). Ensuring stable Vcc is critical for reliable flip-flop operation.

- Display Brightness: The 330 Ω resistors limit segment current to approximately 15 mA at 5V, within the maximum rating of standard LED displays.

The project successfully reinforced classroom theory by applying counter design, cascading, gating, and display interfacing in a practical simulation environment.

# 10. Conclusion

This laboratory project successfully demonstrated the design and simulation of a digital stopwatch using Digital Logic Design principles. Through the implementation of BCD decade counters (74HC90), MOD-6 reset logic, BCD-to-7-segment decoders (74LS47), and start/stop control logic in NI Multisim 14, a fully functional stopwatch counting from 00:00 to 59:59 was realized.

Key skills and concepts acquired through this project include:

- Design and cascading of BCD and MOD-N counters.

- Implementation of combinational reset logic for custom modulus counters.

- Interfacing digital counter outputs with seven-segment LED displays.

- Clock gating for start/stop control using flip-flops and AND gates.

- Practical use of Multisim 14 for schematic entry and digital circuit simulation.

The project outcomes align with the learning objectives of the DLD course and demonstrate the practical applicability of digital logic in real-world timing systems.